Yuma Mechanical Construction Notes :

- Order all the raw metal.  A consolidated list can be found in the file Yuma40_Parts.xlsx under the tab “MechConsolidated”

- Collect all the raw metal for the mechanical construction and cut into their respective parts.  These are listed in Yuma40_Parts.xlsx under tab Yuma40_Mechanical

- For the Base, you will need :

  - Aluminum base

  - Drills :   #21     0.25”     11/32”     #27     #36

  - Taps :     10-32     6-37

  - Notes : most of the drilling is from the top or through.  Hole A is only from the bottom !

- Towers :   Tower-1

  - Aluminum bar:  2” x 0.25” x 2” length, milled to 1.678” length

  - Drills :  #27  (0.144”)           #9 (0.196”)

- Towers :   Tower-2

  - Aluminum bar:    1” x 0.5” x 4.000” length

  - Drills:     #36        #21

  - Taps:     6-32      10-32